Connected Component labeling (CCL)

- Goal: give every separate white blob its own unique number (label)

- Classic algorithm = two-pass algorithm

1st pass- scan left-to-right, top-to-bottom

  - If pixel is white and has no labeled neighbours – give it a new provisional label

  - If it touches already labeled neighbours – give it the smallest label among them

  - If neighbours have different labels – those labels are equivalent – record the equivalence

2nd pass – equivalence resolution

  - Use union-find (with path compression + union-by-rank) or flatten the table

  - Every provisional label is mapped to on=ke final/canonical label

  - Second pass – replace every pixel with its final label

- 4-connectivity vs 8-connectivity

  - 4-connected: only up, down, left and right – diagonal white pixels are NOT connected

  - 8-connected: includes diagonals – almost always used for real blobs

Shape descriptors (the calculations)

Thresholding & Otsu’s method

- Thresholding – converts grayscale to binary

- Histogram = graph of how many pixels have each brightness (0-255)

- Bimodal histogram = two big peaks (dark background + bright object)

- Otsu’s method automatically find the best threshold by maximising the variance between the two groups – the threshold almost always lands in the deep valley between the two peaks

- Works perfectly when object and background have very different brightness – fails on low-contrast or messy images

Convolution basics

- A filter/kernel  slides over the image

- At every position: multiply kernel values with pixel values – add them up – put result in output image

- Edge filters produce negative numbers – to show as normal picture we:

  - Take absolute value, or

  - Add 128 and clip to 0-255

- Padding:

  - No padding (“valid”) – output image gets smaller

  - Zero-padding (“same”) – output image stays same size as input

Morphology – what each does

- Erosion: shrinks white objects, kills tiny specks and thin parts

- Dilation: grows objects, fills tiny holes

- Opening (erosion to dilation): removes small white noise and spikes keeps big object size

- Closing (dilation to erosion) fills small black holes and gaps, keeps big objects size

Color spaces

- Thresholding is converting grayscale to binary

| Channel | What it means | Lighting changes affects it |
| --- | --- | --- |
| RGB | Amount of Red, green and blue | Yes – shadow changes all three a lot |
| Hue | The actual colour (0° = red, 120° = green, 240° = blue) | Almost unchanged by shadows or brightness |
| Saturation | How “pure” or “vivid” the colour is (0 = grey, 1 = pure colour) | Stays very stable under normal lighting changes |
| Value | Overall brightness | Changes a lot with light/shadow |

- Why RGB is terrible for colour segmentation

  - Same red object in light and in shadow has totally different R,G,B number

  - Threshold fails

- HSV is perfect:

  - Same red object keeps almost the same Hue (and usually similar saturation) – easy to threshold

- Brightness-based thresholding

  - Thresholding only on V (or on grayscale) works when object and background differ in brightness, but fails when they have similar brightness but different colour (ex, yellow test on white paper)

Camera & Measurement Fundamentals

- Field of view (FOV):

  - Wider FOV – shorter focal length or larger sensor

  - Narrower FOV – longer focal length or smaller sensor

- Image brightness:

  - Lower f number (ex f/1.8) = much brighter image

  - Higher f number ( ex f/11) = darker image

- Depth of field:

  - Lower f number = shallow depth of field (blurry background)

  - Higher f number = everything in focus

- Pixels – real mm:

  - Real size (mm) = pixel_size x calibration factor (mm/pixel)

  - mm/pixel = known_distance_in_mm / measured_pixels

- Typical calibration:

  - Photograph a ruler or calibration plate – mm/pixels = actual_mm / pixel_count

Part B:

- Why does an automatic threshold (Otsu) fall in the valley of a bimodal histogram?

Answer (3–4 sentences – write this):“Otsu’s method looks for the threshold that makes the foreground pixels and background pixels as different as possible from each other (maximises the distance between the two group averages).

When the histogram has two clear peaks (bimodal), the biggest possible separation between the two groups only happens when the threshold sits in the deep valley between the peaks.

If you move the threshold into either peak, you start mixing object pixels with background pixels, so the separation gets worse.

That is why the optimal Otsu threshold almost always lands right in the valley.”2. Post-labeling filtration – area-based filtering (why we do it)

Answer (3 sentences – write this):“After connected component labeling we usually get hundreds of tiny components (3–20 pixels) caused by sensor noise, dust, or imperfect thresholding.

These tiny blobs make area, perimeter, and compactness values that jump all over the place and create false objects.

By simply removing every component whose area is smaller than a chosen threshold (e.g. 30–100 pixels), we eliminate almost all noise and the remaining real objects now have stable, repeatable shape descriptors.”